MOBILE LIFTING EQUIPMENT - DAILY INSPECTION LIST

| PLANT DESCRIPTION: | WEEK COMMENCING | PLANT SERIAL NO. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| Check all ropes are correctly positioned on their sheaves and drums have not been displaced |  |  |  |  |  |  |  |  |  |  |
| Visually check that no electrical equipment is exposed to contamination by oil, grease, water or dirt |  |  |  |  |  |  |  |  |  |  |
| Check that no fluid loss has occurred, such as lubricating/hydraulic oil and coolant by visual inspection |  |  |  |  |  |  |  |  |  |  |
| Check operation of all limit switches/cut outs and dead man’s handle or lever |  |  |  |  |  |  |  |  |  |  |
| Check rated capacity indicator is set for its correct duty and manufacturers daily test is carried out |  |  |  |  |  |  |  |  |  |  |
| Check load radius scale is appropriate to the jib configuration |  |  |  |  |  |  |  |  |  |  |
| Without a load, vary the load lifting attachment radius to check for correct movement |  |  |  |  |  |  |  |  |  |  |
| Check correct air pressure maintained in pneumatic control systems, e.g. brakes |  |  |  |  |  |  |  |  |  |  |
| Check lights, windscreen wiper(s) and washers operate efficiently |  |  |  |  |  |  |  |  |  |  |
| Check security of wheels and tyre condition |  |  |  |  |  |  |  |  |  |  |
| Check function of all crane controls without load |  |  |  |  |  |  |  |  |  |  |
| Audible warning devices working satisfactory |  |  |  |  |  |  |  |  |  |  |
| Ensure security of loose items |  |  |  |  |  |  |  |  |  |  |
| Access and egress are adequate |  |  |  |  |  |  |  |  |  |  |
| Appropriate safety and fire fighting equipment is available |  |  |  |  |  |  |  |  |  |  |
| Path of crane is free from obstruction |  |  |  |  |  |  |  |  |  |  |
| Operators Initials |  |  |  |  |  |  |  |  |  |  |

ALL FAULTS MUST BE REPORTED IMMEDIATELY TO THE SUPERVISOR/MANAGER

| Fault Reported | Date | Action Taken | Signed Off By |
| --- | --- | --- | --- |

MOBILE LIFTING EQUIPMENT - WEEKLY INSPECTION LIST

| PLANT DESCRIPTION: | WEEK COMMENCING | PLANT SERIAL NO. |  |  |
| --- | --- | --- | --- | --- |
| The r.c.i. in accordance with the operating instructions |  |  |  |  |
| Visually inspect all ropes for broken wires, flattening, basket distortion or other signs of damage, excessive wear and surface corrosion |  |  |  |  |
| All rope terminations, swivels, pins and retaining devices and check all sheaves for damage, worn bushes or seizure |  |  |  |  |
| Inspect the structure for damage, e.g. missing and bent bracings on bridges and strut jibs, bulges, indentations and unusual rubbing marks on telescopic jibs, cracked welds and loose bolts and other fasteners |  |  |  |  |
| Hook(s) and other load lifting attachments, safety catches and swivel(s) for damage, free movement or wear. Check the hook shank thread and securing nut for undue movement, which may indicate wear or corrosion |  |  |  |  |
| Operation and adjustment of controllers |  |  |  |  |
| On hydraulic machines check for creep of hydraulic rams |  |  |  |  |
| Effectiveness of brakes and clutches |  |  |  |  |
| On wheel mounted mobile cranes check tyres for pressure as well as damage and wear on walls and tread. Also check wheel nuts for tightness |  |  |  |  |
| Slew lock if fitted |  |  |  |  |
| On rubber tyred cranes check steering, brakes (foot and parking), lights indicators, horn, windscreen wipers and washers |  |  |  |  |
| Operators Initials |  |  |  |  |

ALL FAULTS MUST BE REPORTED IMMEDIATELY TO THE SUPERVISOR/MANAGER

| Fault Reported | Date | Action Taken | Signed Off By: |
| --- | --- | --- | --- |