Roof Work - Site Safety Checklist

| Address of Site |  | Date of Inspection |  |
| --- | --- | --- | --- |
| Name and position of person carrying out inspection |  | Signature |  |
| Risk Activity | Controls | Y/N | Comments |
| Access to Roof (eg risk of falling when getting on/off ladders at eaves etc) | Have access points been discussed with roofing gang to reduce travel over roof? Is mobile access equipment used? How is access gained? If ladders are used as opposed to towers what are the safe means available for hoisting smaller components not craned up? If long ladders are used how are they made stable? |  |  |
| Edge Protection (Risk of falling outside building frame, gable ends in particular) | Is suitable edge protection provided at: Eaves Gable Ends Other open edges Ridges unless work is progressing at same rate on both roof slopes |  |  |
| Use of Crane (Risk of falling from steel frame when unslinging) | Who accepts load and unslings? How are they protected from falling? Is mobile access equipment used/necessary? |  |  |
| Materials Handling (Risk of falling when walking past open edges to fetch materials) | How are roofing packs loaded out onto roof? (eg splitting packs/back loading onto completed roof reduces travel past open edges) Is travel over roof reduced to a minimum? |  |  |
| Gutter Fixing (Risk of falling from both sides of gutter) | How are workers protected from falling? (peripheral edge protection will not be sufficient at this phase) Is mobile access equipment provided? If not why not? If a running line/harness and lanyard used who erected and tested? |  |  |

| Risk Activity | Controls | Y/N | Comments |
| --- | --- | --- | --- |
| Leading Edge Protection (Risk of falling from partly completed roof) | What is the method for laying the first sheet? How is leading edge protection achieved? Are safety nets necessary? How are stagings made stable? Are stagings of adequate width? Are stagings provided with adequate handrails? What method is used to move stagings? Are there any fragile areas on the roof? (eg roof lights) if so how are they protected? What determines when and if liner panels can be treated as non-fragile? Is the roof zoned into working and non-working areas to reduce the number of roof lights that need to be protected? |  |  |
| Risk of slipping or being blown off edges | What precautions are taken? |  |  |
| Personal Protective Equipment (PPE) | List the PPE required (eg hard hat, safety harness, safety footwear, ear protection, high visibility clothing, gloves, etc). Are all employees wearing suitable PPE? If not list names of individuals and which PPE is not being worn. |  |  |
| Systems Control | Communications Has the method statement been communicated to and agreed with the roofing gang? Supervision What are the arrangements for ensuring that work proceeds according to method statement? Who is responsible? Modification What are the arrangements for agreeing modifications to the method statement? How are modifications communicated to roofing gang? Validation Is the person carrying out this site safety inspection satisfied that the systems of work that are in place are safe? If not indicate why on page 3. |  |  |

Remedial Action

| Defects Found | Action Taken |
| --- | --- |