On-Site Construction Health and Safety Checklist

| Client (if applicable): |  |
| --- | --- |
| Site address: |  |
| Details of work to be undertaken: |  |
| Approximate duration of work: |  |

This checklist identifies some of the common hazards found on construction sites and for maintenance work. It is not intended to be an exhaustive list as the hazards will vary on each project. This list may be of assistance when carrying out the risk assessment for the project, as well as for inspecting the site during construction operations. Regular inspections are important to maintain a safe and healthy workplace. Where defects are identified, action should be taken to correct the defects immediately, or work should be stopped until the necessary repairs have been made. In order to demonstrate that inspections are being carried out, a record should be maintained, e.g. site inspection form or a record in site diary etc.

| 1. Safe Places of Work |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |
| a) | Can everyone reach their place of work safely, ie are roads, gangways, passageways, passenger hoists, staircases, ladders and scaffolds in good condition? |  |  |  |
| b) | Are there guard rails or other suitable protection in place to stop falls from open edges, e.g. scaffolds, excavations etc? |  |  |  |
| c) | Are holes and openings provided with a guard-rail or covered by a secure cover in order to prevent falls? |  |  |  |
| d) | Are structures stable, adequately braced and not overloaded? |  |  |  |
| e) | Are all working areas and walkways level and free from obstructions such as stored material and waste? |  |  |  |
| f) | Is the site tidy and all materials safely stored? |  |  |  |
| g) | Are there proper arrangements in place for the collection and disposal of waste materials? |  |  |  |
| h) | Is there adequate lighting on the site? |  |  |  |
| i) | Is additional lighting available for work after dark or inside buildings? |  |  |  |
| 2. Scaffolds |  |  |  |  |
|  |  | Yes | No | N/A |
| a) | Are scaffolds erected, altered and dismantled only by competent persons? |  |  |  |
| b) | Is there safe access to the scaffold platform? |  |  |  |
| c) | Are all uprights provided with base plates (and where necessary timber sole plates) or prevented in some other way from slipping or sinking? |  |  |  |
| d) | Are all the uprights, ledgers, braces and struts in position? |  |  |  |
| e) | Is the scaffold adequately tied into the structure to prevent collapse? |  |  |  |
| f) | Are there adequate guard rails and toe boards at every edge from where a person could fall 2 metres or more? |  |  |  |
|  | i) Are the toe boards at least 150 mm in height? |  |  |  |
|  | ii) Is the upper guard rail at least 910 mm above the working platform? |  |  |  |
|  | iii) Are intermediate guard rails or brick guards provided to ensure that there is no unprotected gap of more than 470 mm? |  |  |  |
| g) | Are the working platforms fully boarded and are the boards arranged to avoid tipping or tripping? |  |  |  |
| h) | Are there effective barriers or warning notices to stop people using an incomplete scaffold? |  |  |  |
| i) | Has the scaffold been designed and constructed to cope with the materials stored on it and are these evenly distributed? |  |  |  |
| j) | Was a hand-over certificate obtained from the contractor who erected the scaffold? |  |  |  |
| k) | Is the scaffold inspected at least once a week by a competent person and are records kept of all inspections? |  |  |  |

| 3. Powered Access Equipment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Yes | No | N/A |
| a) | Has the equipment been erected by a competent person? |  |  |  |  |
| b) | Is fixed equipment, eg mast climbers, rigidly connected to the structure against which it is operating? |  |  |  |  |
| c) | Does the platform have adequate guard rails and toe boards or other barriers to prevent workers and materials falling off? |  |  |  |  |
| d) | Have precautions been taken to prevent people from being struck by the moving platform, projections from the building or falling materials, eg barrier around base? |  |  |  |  |
| e) | Are the operators trained and competent? |  |  |  |  |
| f) | Is the power supply isolated and the equipment secured at the end of the working day? |  |  |  |  |
| 4. Ladders |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Are ladders the right means of access for the job? |  |  |  |  |
| b) | Are all ladders in good condition? |  |  |  |  |
| c) | Are they adequately secured to prevent them from slipping sideways or outwards? |  |  |  |  |
| d) | Do ladders rise about 1 m above their landing place, or is there some other handhold available |  |  |  |  |
| e) | Are the ladders positioned so that the user doesn’t have to over-stretch? |  |  |  |  |
| 5. Roof Work |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Is edge protection provided which prevents persons or materials falling from the roofs? |  |  |  |  |
| b) | Do the roof battens provide safe hand and foot holds? |  |  |  |  |
|  | If not, are crawling ladders or boards provided? |  |  |  |  |
| c) | During industrial roofing, are precautions taken to stop people from falling from the leading edge of the roof, or from thin or partially fixed sheets which could give way? |  |  |  |  |
| d) | Are suitable barriers or secure coverings provided where people pass or work near fragile materials such as asbestos cement sheets and glass and roof lights? |  |  |  |  |
| e) | Are crawling boards provided where people must work on fragile materials? |  |  |  |  |
| f) | Are people excluded from the area below the roof work:? |  |  |  |  |
|  | If this is not possible, have additional precautions been taken to prevent any debris from falling onto the area below? |  |  |  |  |
| 6. Excavations |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Is appropriate means of support, eg trench sheets or boxes, timber and props etc, provided? |  |  |  |  |
| b) | Is there a safe method of work used for putting in the support, ie one which does not rely on people working in the unsupported trench? |  |  |  |  |
| c) | If the sides of the excavation are sloped back or battered, is the angle of batter sufficient to prevent collapse? |  |  |  |  |
| d) | Is there safe means of access into the excavation, ie secured ladder? |  |  |  |  |
| e) | Are guard rails or other protection provided in order to prevent persons from falling into the excavation? |  |  |  |  |
| f) | Does the excavation affect the stability of neighbouring structures? |  |  |  |  |
| g) | Are stacked materials, spoil or plant stored away from the edge of the excavation so as not to cause a collapse of the sides? |  |  |  |  |
| h) | Is the excavation inspected by a competent person prior to the start of every shift? |  |  |  |  |
| 7. Manual Handling |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Are hoists, telehandlers, wheelbarrows and other plant or equipment used so that manual handling of heavy objects and materials is minimised? |  |  |  |  |
| b) | Are materials such as cement ordered in 25 kg bags? |  |  |  |  |
| c) | Can the handling of heavy building blocks be avoided? |  |  |  |  |
| 8. Hoists |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Is the hoist protected by a substantial enclosure to prevent someone from being struck by any moving part of the hoist or falling down the hoistway? |  |  |  |  |
| b) | Are gates provided at all landings? |  |  |  |  |
| c) | Are the gates kept shut except when the platform is at the landing? |  |  |  |  |
| d) | Are the controls arranged so that the hoist can be operated from one position only? |  |  |  |  |
| e) | Is the hoist operator trained and competent? |  |  |  |  |
| f) | Is the hoist’s safe working load clearly marked? |  |  |  |  |
| g) | If the hoist is for materials only, is there a warning notice on the platform or cage to stop people riding in it? |  |  |  |  |
| h) | Is the hoist examined weekly, and thoroughly examined every six months by a competent person, with records being maintained? |  |  |  |  |
| 9. Cranes and Lifting Appliances |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Is the crane on a firm level base? |  |  |  |  |
| b) | Is the safe workings load known and the lifting operation planned out in advance? |  |  |  |  |
| c) | If the crane has a capacity of more than 1 tonne, does it have an automatic safe load indicator fitted? |  |  |  |  |
| d) | If an excavator is used as a crane, does it have the necessary check valves fitted and its safe working load clearly marked? |  |  |  |  |
| e) | Are all drivers and banksmen trained and competent? |  |  |  |  |
| f) | Are the correct chains, slings, strops etc being used and are they in good condition? |  |  |  |  |
| g) | Are cranes inspected weekly and thoroughly examined every 14 months? |  |  |  |  |
| h) | Are all inspections recorded and does the crane have a current test certificate? |  |  |  |  |
| 10. Plant and Machinery |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Are all dangerous parts, eg exposed gears, chain drives etc, guarded? |  |  |  |  |
| b) | Are guards secure and in good condition? |  |  |  |  |
| c) | Is the machinery maintained in good repair and are all safety devices operating correctly? |  |  |  |  |
| d) | Are operators trained and competent? |  |  |  |  |
| 11. Electricity |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Do all tools and equipment operate at 110 volts or lower? |  |  |  |  |
| b) | Where mains voltage has to be used, are trip devices, eg residual current devices, provided for all equipment? |  |  |  |  |
|  | Are they checked daily? |  |  |  |  |
| c) | Are cables and leads protected from damage by sheathing or protective enclosures, or positioned away from sources of damage? |  |  |  |  |
| d) | Are all connections to the system properly made with suitable plugs being used? |  |  |  |  |
| e) | Is there a system for user checks, formal visual inspections by site manager and combined inspection and test by competent persons for all tools and equipment? |  |  |  |  |
|  | Are records maintained of all examinations and tests? |  |  |  |  |
| f) | Is there a safe system of work to avoid damage to, or danger from, underground services? |  |  |  |  |
|  | Have plans been obtained, services traced eg with cable locators, trial holes dug by hand and safe digging practices followed? |  |  |  |  |
| g) | Have the appropriate precautions been taken in relation to overhead services? |  |  |  |  |
| 12. Traffic and Vehicles |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Have separate pedestrian and vehicle access points and routes around the site been provided? |  |  |  |  |
|  | If not, are vehicles and pedestrians dept separate whenever possible? |  |  |  |  |
| b) | Have one-way systems or turning points been provided to minimise the need for reversing? |  |  |  |  |
| c) | Where vehicles have to reverse, are they controlled by properly trained banksmen? |  |  |  |  |
| d) | Are vehicles maintained; do the steering, handbrake, footbrake, flashing beacons etc work properly? |  |  |  |  |
| e) | Have drivers received proper training and authorisation to operate vehicles or plant? |  |  |  |  |
| f) | Are vehicles properly loaded and loads secured? |  |  |  |  |
| 13. Fire and Emergencies |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | General |  |  |  |  |
|  | i | Have emergency procedures been developed, eg evacuation in event of a fire, bomb warning etc? |  |  |  |
|  | ii | Do workers and visitors know the procedure? |  |  |  |
|  | iii | Is a register kept of all persons on site so that everyone can be accounted for? |  |  |  |
|  | iv | Is there a means of raising the alarm and is it effective? |  |  |  |
|  | v | Are there adequate escape routes? |  |  |  |
| b) | Fire |  |  |  |  |
|  | i | Is the quantity of flammable material on site kept to a minimum? |  |  |  |
|  | ii | Is there proper storage for flammable liquids and gases, eg liquefied petroleum gas and acetylene? |  |  |  |
|  | iii | Are suitable containers used for storing and transferring flammable materials? |  |  |  |
|  | iv | Is smoking banned in areas where flammable liquids and gases are stored and used? |  |  |  |
|  | v | Are other potential ignition sources also prohibited? |  |  |  |
|  | vi | Is waste regularly removed and are adequate bins or skips provided? |  |  |  |
|  | vii | Have fire points been established and are the correct type of fire extinguishers provided? |  |  |  |
|  | viii | Is all hot work, eg flame cutting and welding, carried out with the necessary precautions in place? |  |  |  |
| 14. Hazardous Substances |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Have all harmful materials, eg asbestos, lead, solvents, paints etc, been identified? |  |  |  |  |
| b) | Have risks to everyone who might be exposed to these substances been assessed? |  |  |  |  |
| c) | Have precautions been identified and put in place, eg use of local exhaust ventilation? |  |  |  |  |
| 15. Noise |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Are breakers and other plant or machinery fitted with silencers? |  |  |  |  |
| b) | Are barriers erected to reduce the spread of noise? |  |  |  |  |
| c) | Is the work sequenced so as to minimise the number of people exposed to noise? |  |  |  |  |
| d) | Is ear protection provided and worn? |  |  |  |  |
| 16. Welfare |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Have suitable toilets been provided and are they kept clean? |  |  |  |  |
| b) | Are there clean wash basins, warm water, soap and towels? |  |  |  |  |
| c) | Is suitable clothing provided for those who have to work in wet, dirty or otherwise adverse conditions? |  |  |  |  |
| d) | Are there facilities for changing, drying and storing clothing? |  |  |  |  |
| e) | Is drinking water provided? |  |  |  |  |
| f) | Is there accommodation provided where workers can sit, make tea and prepare food? |  |  |  |  |
| g) | Are suitable first aid facilities provided? |  |  |  |  |
| 17. Protective Clothing |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Has adequate protective equipment, eg hard hats, safety boots and gloves, been provided? |  |  |  |  |
| b) | Is a register maintained of all protective equipment issued? |  |  |  |  |
| c) | Is the equipment worn by all who need to? |  |  |  |  |
| 18. Protection of the Public |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Is the site fenced or otherwise protected to prevent access: |  |  |  |  |
| b) | At the end of the working shift, is an inspection made to ensure that the site is left in a safe condition? |  |  |  |  |
| 19. Training, Instruction and Supervision |  |  |  |  |  |
|  |  |  | Yes | No | N/A |
| a) | Have workers been appropriately trained for the work they are carrying out? |  |  |  |  |
| b) | Have all persons received a site induction in relation to the specific arrangements for health and safety on the site? |  |  |  |  |
|  | Are records maintained? |  |  |  |  |
| c) | Are method statements and safe systems of work provided to workers? |  |  |  |  |

Additional Action Recommended to Control / Reduce Risks

Print Name: …………………………………………………………………………………….

Date: ……………………………………………………………………………………………

Signature: ………………………………………………………………………………………

Position in Company: ………………………………………………………………………….