- Examination and Testing of Portable and Transportable Electrical Equipment - GUIDELINES FOR USE OF PORTABLE ELECTRICAL EQUIPMENT - General The use of an RCD device will improve the safe use of portable electrical equipment. - A. User Checks (Visual) Serious Defect - Equipment to be withdrawn immediately. Users must carry out a visual inspection of plugs and leads before using any item of electrical equipment which is earthed or normally hand held (see Table in 5(d)). Look out for: - An electrical test pass sticker, green pass, red fail. - Damage to the cable (apart from light scuffing) e.g. kinks, cuts, abrasions. - Damage to the plug e.g. cracked casing, bent pins. - Equipment with intermittent fault. - Inadequate joints e.g. taped "chocolate block" connectors. - The outer sheath of the cable should be effective and secured where it enters the plug or equipment. There should be no exposed wires. - Damage to external casing of equipment e.g. loose parts, or screws. - Evidence of overheating e.g. burn marks or discoloration. - Has equipment been subjected to conditions for which it is not suitable e.g. is it wet or contaminated. - Protective cable grommets, which protect cables passing through sharp metal cases - these should be effective. Damaged or loose grommets are a frequent cause of premature cable failure. - B. User-Safe Operating Guidelines - Do not attempt to repair or make alterations to equipment and never remove permanently fixed covers from appliances. - Report any defects and label equipment "out of order". - Do not use damaged or defective equipment and equipment which has failed a test. - Remember to switch off equipment and unplug before cleaning or fitting new parts. - Always remove plugs by holding the plug and not pulling on the cord. - Ensure that cables are not squashed under furniture, doors or touching warm radiators. - Do not overload socket outlets. Avoid or minimise the use of adapters. Overloaded sockets can lead to fire hazards. If necessary, use a fused 13 amp block extension board to BS 1363A. - Do test the residual current device if fitted. - Do not use electrical equipment in a wet environment. Avoid drinking near electrical equipment. - Do not bring personal items of equipment into the work place without first having it checked by your Departmental Safety Officer. - Do find out how to deal with an electrical shock incident. Your Departmental Safety Officer and / or first-aider will be able to advise. - Do find out the correct fire extinguisher to use on an electrical fire. - Plugs & Fuses You do not need to dismantle the plug as part of a user check, or have it dismantled for you, unless you are suspicious of its condition. If you do so and find a fuse has been illegally "wired out" with a nail or a paper clip, or the fuse rating is simply too high, then remove it from use and have it checked. - Table 2 Fuse ratings examples: Normal Amps; Example equipment; Normal Wattages. 3 amp; Desk lamp, radio, hi-fit; Up to 600 watts. 5 amp; Iron, TV; Up to 1,000 watts. 7 amp; Hair dryer; Up to 1,400 watts. 13 amp; Kettle; Up to 3,000 watts.