Tool Box Talk

Safe use of “A” Frame Ladders

1.1     Introduction

- “A” Frame Ladders are one of the most misused piece of equipment within the workplace.

- Remember, The vast of majority of ladder accidents occur due to “A” Frame Ladders being unstable

- This talk will cover: Pre use checks, use of ladders and associated hazards.

  - Pre use checks

- Ladders should be stored correctly and inspected regularly

- Check for cracks or corrosion of metal fittings

- Are the steps bent or damaged in any way / are steps missing

- Report defects, label as defective and remove from use

  - Use of Ladders

- Work should only be carried out from a “A” Frame Ladder when the job is of a short duration and is safe to do so

- Ladders should be placed on a firm and level surface

- The supporting struts must be locked in place as well as the load platform

- Use both hands on the stiles to go up and down the ladders

- Don’t lean outwards or sideways, always face the ladder when climbing up or coming down

- Ensure footwear is suitable for purpose and steps are free from mud, grease etc

DO YOU HAVE ANY QUESTIONS ?