Permits to Work - Typical Sequence of Operations

Request for work to be done

Determine type of work and the need for a permit

No

Yes

Comply with other requirements

Work cannot be done without a permit being issued

Refer to manager in charge of permits to work

Work to be performed in house

Work to be performed by external contractor

Specialist equipment involved.  Inspect, test and record

Check personal training and authorisations

Inspect site and satisfy compliance with statutory obligations

Isolate plant, make area safe, withdraw personnel, provide additional safety related equipment ie fire safety

Issue permit - including open and closure periods

Work completed within time limit

Work not completed within time limit

Refer back to issuing manager

Refer back to issuing manager

Inspect area and make safe to commence work

Inspect / clarify

Issue an extension

Close permit

Return to service

Request for work to be done in

a confined space

Work can be done without entry

Withdraw from service

Authorise work/prohibit entry

Work cannot be done without entry

Refer to supervisor in charge of

permits-to-work

Work to be done with breathing

apparatus

Work to be done without breathing

apparatus

Check that the wearing of breathing

apparatus is practicable inside the

plant and during access and egress

Withdraw from service

Withdraw from service

Isolate plant or section concerned

Check no source of ingress of

dangerous material (eg steam)

Isolate plant or section concerned

Withdraw from service

Check no source of ingress of

gas/fumes, etc

Remove sludge and/or deposits if

present

Clean inside surfaces by appropriate

method/ventilate thoroughly

Test atmosphere inside

Certify as being safe for entry

without breathing apparatus

Not safe for entry without breathing

apparatus

Refer back for further

cleaning/purging as necessary

Approve entry with specified

breathing apparatus, life line, etc

Specify:	Protective clothing if needed

Second man outside

Rescue equipment

Reviving apparatus

Issue permit-to-work certificate/specify time limit

Work not finished inside time limit

Refer back to supervisor for renewal or new permit

Work finished inside time limit

Warn personnel return permit for cancellation

Return to service