SAFE SYSTEMS OF WORK

PARKING OF LARGE GOODS VEHICLES

1.	Vehicles must be parked on firm level ground that is capable of supporting the trailers landing legs, preferably in a designated parking area.

2.	When this is not possible and a vehicle has to be parked on a gradient, vehicles must have their brakes applied, and where safe to do so, be left in gear and wheel-chocks used.

Key Steps for Safe Coupling of Tractor Units & Trailers:

- Check the trailer parking brake has been applied

- Slowly reverse the tractor unit under the trailer, listening for the locking mechanism to engage

- Check to ensure that the fifth wheel locking mechanism has engaged (usually done by trying to drive forward in a low gear)

- Apply the parking brake of the tractor unit

- Visually check the engagement of the fifth wheel, apply any safety clips that are required depending on the type of mechanism

- Reconnect all air lines and electrical supplies to the trailer.

- Retract the landing legs on the trailer unit and secure the handles in place

- Release the trailer parking brake.

- Fit number plate and warning plates to the trailer and check the lights are working

Key Steps for Safe Uncoupling of Tractor & Trailers

- Apply the parking brake on the tractor unit

- Leave the cab of the vehicle by facing the vehicle and using the grab handles and steps provided.

- Remove the number plate and warning plates from the trailer.

- Apply the trailer parking brake.

- Extend the landing legs on the trailer so that they are in firm contact with the ground and secure the handles in place.

- Disconnect all air lines and electrical supplies from the cab.

- Remove any safety clips that are securing the fifth wheel and release the locking mechanism.

- Release the tractor units parking brake and slowly drive the unit forward, releasing the fifth wheel and uncoupling the trailer.

- Stop the tractor unit and apply the parking brake. Walk back to the trailer and ensure that the trailer is secure.