Safe System of Work

Mobile Tower Scaffold

- Only competent and trained personnel to erect mobile tower scaffold

- Always erect on firm level ground

- DO NOT extend the scaffold above the recommended height

- Access ladders should always be provided on the inside

- Before using the scaffold:

- ensure scaffold is vertical

- wheel brakes are on

- scaffold is secured or stabilised by outriggers or is tied to the building / structure

- When working at heights in excess of 1 metres or as identified by the Risk Assessment ensure guard rails and toe boards are fitted around the work platform

- When moving a mobile scaffold:

  - check for power lines or other overhead obstructions

  - check the ground for obstructions and that it is firm and level

  - push or pull from the base

  - NEVER move the scaffold while there are people or materials on the work platform

  - NEVER move the scaffold in extreme windy conditions

- Mobile scaffold should be inspected:

  - before its first use

  - after any alterations

  - after any event likely to have affected its stability ie weather conditions

  - inspected again – not exceeding seven days

  - visually inspected at the beginning of each shift

- Always IF IN DOUBT – STOP WORK AND SEEK ADVICE